eVault 18kWh MOD Tool Instructions

Please note we do not usually use this tool. 
Please contact Fortress technical support for help reading error codes. You can book an appointment in advance by logging into our support portal at https://support.fortresspower.com

BMS Save history data guide

1. Prepare tools: Computer, RS485 to USB converter, RJ45 cable，

2.Start-up battery and connect, and connect communication port（Pin3, 5）to USB converter.

| RS485 Port |  |
| --- | --- |
| Pin No. | Definition |
| 1 | CANH |
| 2 | CANL |
| 3 | RS485-A |
| 4 | NC |
| 5 | RS485A-B |
| 6 | ISO-GND |
| 7 | RX |
| 8 | TX |

3. Then run the BMS debug software → turn to “STORAGE” page → click “Read time” → write and then set “Read Time” and set ”Start time”→ click “save Record”.

If set long time duration, the BMS will download latest 500 lines history data, and that will take about 5min.