Release Notes

eVault Max

Firmware revision:

| Revision | Editor | Date | Changes from previous Revision |
| --- | --- | --- | --- |
| HEBMS0121430PW01T |  | 5/24/2021 | Schneider and Solark Protocol |
| HEBMS0121521PW01T |  | 5/28/2021 | Communication RS485 and CAN to inverter. Starting byte debugged |
| HEBMS0121531PW01T |  | 5/31/2021 | BMS: add fortress standard Modbus. |
| HEBMS0121602PW01T |  | 6/2/2021 | Updated com protocol on ID 3 to display SOC and Voltage on Fortress modbus |
| HEBMS0121612PW01T |  | 7/6/2021 | Protocol ID-1 fortress power modbus，9600bps， Completed, the attachment program needs to be updated. Protocol ID-2 SMA canbus，500kbps The estimated submission process time is 6-30 Protocol ID-3 fortress power CAN，500kbps Estimated submission time: 6-23 Protocol ID-4 victron CAN，250kbps， Estimated submission time: 7-15 Protocol ID-5 Schneider Modbus，19200bps， Completed, the attachment program needs to be updated. |
| HEBMS0121617PW01T |  | 6/17/2021 | Official release can use USB |
| HEBMS0121623PW01T.s19 |  | 6/24/2021 | SMA added—Non functional. DO NOT USE USB |
| HEBMS0121624PW01T_Test.s19 |  | 6/24/2021 | - Internet Comm to 1 sec - Increasing abnorminal communication ping to 3 min - Records comm failures and BMS resets |
| HEBMS0121713PW01T |  | 8/23/2021 | Reduces chance of BMS jamming with multiple units using USB drive |
| HEBMS0121818PWXXT |  | 8/26/2021 | SOC control for Charge allowance set at 95% SOC After reaching full charge Soc Calibration points - (CellV == 2.8 && SOC > 10%) SOC = 0% - (CellV == 3.4 && SOC < 80%) SOC = 90% |
| 121827 |  |  | - Added Victron Closed Loop Communication - Fixed SMA CAN bugs SMA Functional |
| 121831 |  |  | Bug fix SOC < 95% and CellV >3.6 at charge cycle induces error |
| 121C01 |  | 12/02/2021 | Schneider Protocol Bugs Addressed |